TASK ORDER DETAIL 	September  23, 2014

USGS CONTRACT:	G10PC00025

CONTRACTOR:	QUANTUM  SPATIAL

TASK ORDER NUMBER:  G14PD01045

TASK NAME:	San Diego CA 2014 Ortho and Lidar Task Order

The Contractor shall furnish all facilities, labor, materials, and equipment, unless specifically identified otherwise, to provide the mapping services and products in accordance with the specifications, terms, and conditions contained in Contract No. G10PC00025 and the following requirements specific to this Task Order and in accordance with Contractor’s Technical and Cost proposal dated 09/19/2014 and revised 09/23/2014 via email in the amount of:

Task Order Fixed Price			$998,607.62

- DESCRIPTION/SPECIFICATIONS/WORK STATEMENT.

The following Section C additional requirements are applicable to this Task Order:

  - Statement of Work (SOW): Reference C.1 of the Contract. San Diego CA 2014 Ortho and Lidar Task Order shall provide for the high resolution 30cm (Nominally 1 foot) and , and 10cm (nominally 4inch) 4-band (8 bit) Red, Green, Blue, and Near Infrared, ortho imagery and QL2 lidar at a resolution of 2 points per square meter. A portion of San Diego County and a small portion of Tijuana, MX (together totaling approximately 1505 square miles), shall be collected at an image resolution of 30cm. Additionally, a portion of San Diego County (approximately 1135 square miles) and Camp Pendleton (approximately 253 square miles) shall be collected at a ground resolution of 10cm and QL2 lidar at a resolution of 2 points per square meter. These areas are further described and identified in Attachment A, and defined in the shape file provided in Attachment B.

Portions of the 10cm imagery resolution area are operated by the Military and subject to resampling of the 10cm imagery to 30cm resolution for release to the public.  Data deliveries to the USGS for review and inclusion in the USGS Data Base and subsequent delivery to clients shall contain a mixture of 30cm and 10cm imagery in the 10cm upgrade area. These exclusion areas are shown in Attachments B and C which are subject to change; but will probably be similar to what is shown; it is likely any change would result in a smaller area being restricted.  Portions of QL2 lidar are also over military property and subject to restrictions concerning public release.  One foot imagery will still be collected over these regions.

Lidar Specifications listed below are based on the “National Geospatial Program Lidar Base Specification Version 1.0”, which is incorporated by reference to this task order.  This specification may be viewed at http://pubs.usgs.gov/tm/11b4/. These lidar specifications are required minimum baseline specifications. In addition to the Specification Requirements, this task order shall meet NEEA QL2. For any item which is not specifically addressed, the referenced Specification Version 1 will be the required specification authority.

It is recommended that the Contractor request a technical briefing prior to submitting a technical/cost proposal for the purpose of reviewing the requirements of this task order.

    - KICK-OFF MEETING: A kick-off meeting shall be held to outline communication procedures that shall be followed for data acquisition with respect to verification of local ground conditions and vegetation requirements. This meeting shall be used as a forum to clarify and resolve collection condition issues. Local contact(s) shall be established to provide ground condition updates. The kick-off meeting shall be held no later than two (2) weeks after contract award and prior to data acquisition.

    - DATA ACQUISITION: This task order is to acquire high resolution ortho imagery which shall cover the area as described and defined in Attachment A and Attachment B of this task order.

      - Image Acquisition: Image data collection shall be with the following minimum collection specifications:

        - Resolution and Accuracy:  Imagery shall be 4-band, 8-bit, natural color and CIR source imagery and shall be of sufficient resolution to support production of digital orthorectified images to a ground pixel resolution of:

          - 30 cm (nominal 1 foot) covering a portion of San Diego County, as well as a small portion to the NW of the county as shown in Attachment A and to the specifications contained in C.1.b below.

          - 10cm (nominal 4 inches) covering, in part, San Diego and numerous other smaller cities in San Diego County as shown in Attachment A and to the Specifications of C.1.b. below. Exclusion are described in Section C.1.c.(xiii) and further described in Attachment B and Attachment C.

        - Acquisition Window: The acceptable window for the acquisition portion of this task shall be between October 7 – December 1, 2014.

        - Ground Sample Density (Resolution): The required ground sample distance shall be 30cm (one foot)  and  10cm (4 inch) as required in C.1.a.(i)(a) above.

        - Time of Day: Image acquisition shall occur when the sun angle is greater than 30-degrees.

        - Acquisition Conditions: Imagery shall be acquired only under conditions free from clouds, cloud and excessive building shadows, smoke, haze, light streaks, snow, foliage, flooding, and excessive soil moisture. Road network within the 10cm area shall be visible with no excessive building lean or shadows obscuring the transportation network.

        - Image Coverage:  The extent of image coverage over the project area shall be sufficient to ensure void areas do not exist within the defined project area. All tiles are required to be filled to the tile boundaries. Void areas within the project area are unacceptable.

        - Non-image data:  Orthoimagery tiles shall not contain any non-image data.  Non-image data includes photographic frame borders, fiducial marks, artifacts, and titling.

        - Calibration:  Reference 3.8.1.2 of the contract.  Aerial Sensors/Camera(s) used to acquire project imagery shall have current USGS certification, or in the case of digital sensors a current Product Characterization Report.

        - Camera Station Control:

          - Airborne GPS: Camera position (ABGPS report, including x, y, z, omega, phi, kappa, and time stamp for each frame) shall be recorded at the instant of exposure with airborne GPS.  Airborne GPS data shall be differentially corrected and organized as individual data sets grouped by corresponding flight line.  Differentially corrected Airborne GPS positional data shall be stored on portable media, in a non-proprietary format mutually agreeable to the Government and the Contractor.  The Contractor shall produce a statistical report summarizing the results of the airborne GPS adjustment.

          - IMU Exterior Orientation Data:  The contractor shall record the camera attitude at the instant of exposure.  The IMU data shall be adjusted and organized as individual data sets grouped by corresponding flight line.  The Contractor shall produce a statistical report summarizing the overall accuracy of the adjusted IMU data.

      - Lidar Acquisition: The contractor shall be responsible for acquisition of lidar data of sufficient density and quality to meet the requirements specified in the referenced Version 1 specification:

        - Collection area: The collection area shall be defined as the Defined Project Area, buffered by no less than 100-meters. The Project Area is defined in “Attachment A – Project Description and Diagram” and further delineated by the ESRI ArcShape file included as “Attachment B – Shape File(s)”

        - Nominal Pulse Spacing: Nominal Pulse Spacing (NPS) shall be no greater than 0.7- meter; assessment to be made against single swath, first return data located within the geometrically usable center portion (typically ~90%) of each swath.

        - Signal Returns The laser system shall be configured to collect multiple echoes per pulse, with a minimum of a first return and a last return and at least one additional intermediate return. All returns captured during acquisition shall be delivered. Return number shall be recorded.

        - GPS Times: shall be recorded as Adjusted GPS Time, at a precision sufficient to allow unique timestamps for each return. Adjusted GPS Time is defined to be Standard (or satellite) GPS time minus 1*109. See the LAS Specification for more detail.

        - Signal Strength: The signal strength (intensity) of each return pulse shall be recorded.

        - Clustering: The spatial distribution of geometrically usable points is expected to be uniform and free from clustering. In order to ensure uniform densities throughout the data set:

          - A regular grid, with cell size equal to the design 2*NPS will be laid over the data.

          - At least 90% of the cells in the grid shall contain at least 1 lidar point.

          - Clustering will be tested against the 1st return only data

          - Acceptable data voids identified elsewhere in this specification are excluded.

        - Control: LIDAR shall be acquired using the following control specifications.

          - Supplemental Ground Control: Differentially corrected GPS Ground Control used to supplement the Airborne GPS positional accuracy.

          - Ground Control Quality Check points:  The Contractor shall collect additional Ground Control Check Points in each project area which shall be delivered in ESRI Arc Shape format and will be used by the Government for validation.

          - A minimum of twenty (20) check points shall be collected uniformly dispersed over the project area in each of the land cover classifications in which there is more than 10% coverage to verify fundamental, supplemental, and consolidated vertical accuracies. In addition – Points shall be collected for those areas which are classified as Urban, even though there is less than 10% coverage.

          - Fundamental vertical accuracy checkpoints should be located only in open terrain, where there is a high probability that the sensor will have detected the ground surface without influence from surrounding vegetation.

          - Checkpoints should be located on flat or uniformly sloping terrain and will be at least five (5) meters away from any breakline where there is a change in slope.

          - The checkpoint accuracy shall satisfy a Local Network accuracy of 5-centimeteres at the 95% confidence level.

          - Most common  land cover categories are (but not limited to):

Bare Earth/Open Terrain

Urban

Tall Weeds/Crops (do not use Low Veg)

Brush lands and Trees (do not use Med Veg)

Forested and Fully Grown (do not use High Veg)

          - Check points shall not be incorporated into the contractor’s vertical solution.

        - Vertical Accuracy Requirements: Lidar collected under this task order shall meet or exceed these vertical accuracies. Assessment procedures shall comply with NDEP guidelines. See C.1.c.(i)(a) below for complete vertical accuracy reporting requirements.

RMSEZ = 9.25 cm

FVA = 18.5 cm 95% Confidence Level (Required Accuracy)

CVA = 26.9 cm 95th Percentile (Required Accuracy)

SVA = 26.9 cm 95th Percentile (Target Accuracy)

        - Positional Accuracy Validation: The absolute and relative accuracy of the data, both horizontal and vertical, relative to known control, shall be verified prior to classification and subsequent product development. A detailed report of this validation is a required deliverable

        - Relative Accuracy Requirements: Relative accuracy shall be <= 6-cm RMSEZ within individual swaths and <=8-cm RMSEZ or within swath overlap (between adjacent swaths)

        - Swath Length: Long swaths (those which result in a LAS file larger than 2GB) shall be split into segments. Each segment shall thenceforth be regarded as a unique swath. Other swath segmentation criteria may be acceptable, with prior approval.

          - Full Swath data shall be delivered Edge data from each swath shall not be trimmed from the delivered data.

        - Overlap: Flight line overlap of 10% or greater, as required to ensure there are no data gaps between the usable portions of the swaths. Collections in high relief terrain are expected to require greater overlap. Any data with gaps between the geometrically usable portions of the swaths will be rejected.

        - Data Voids:  Data Voids [areas => (4*NPS)2, measured using 1st-returns only]  within a single swath are not acceptable, except:

          - where caused by water bodies;

          - where caused by areas of low near infra-red (NIR) reflectivity such as asphalt or composition roofing; or

          - where appropriately filled-in by another swath.

    - DIGITAL ORTHO IMAGE PRODUCTION: All Ortho Images shall be produced consistent with the following requirements:

      - Image Mosaicking:  Orthoimagery may be created using multiple digital images (“chips”) to produce the final product.  Specular reflections and other artifacts should be minimized, especially in developed areas, by patching the area using chips from adjacent imagery.

        - Radiometry Balance: When a mosaic of two or more chips is made, the brightness and color values of the other chips will be adjusted to match that of the principal chip.  The seam lines between the overlapping chips shall be chosen to minimize tonal variations.  Localized adjustment of the brightness and color values will be done to reduce radiometric differences between join areas.  Changes in color balance across the project, if they exist, shall be gradual. Abrupt tonal variations between tiles are not acceptable.

        - Radiometric Resolution: All imagery that contains both natural color and near-IR shall be in accordance with Section 6, RGB Full Color Images, of the TIFF Specification (http://www.remotesensing.org/geotiff/spec/geotiffhome.html), and shall have the bands saved in the following order: Red, Green, Blue, and Infrared.

        - Digital orthophoto characteristics:  Relative join (misalignment) of transportation features between adjacent image chips/tiles shall not exceed 3 pixels. Orthophotos shall be tonally balanced to produce a uniform contrast and tone across the image tiles of the entire project.

          - Changes in color balance across the project, if they exist, shall be gradual. Abrupt tonal variations between tiles are not acceptable. Building tilt shall be corrected to the extent that transportation features are not obscured.

          - Ground features appearing in the orthophoto imagery, such as building roof tops, water towers, and radio towers, shall not be clipped at seam lines or between individual tiles.

      - The contractor shall deliver: uncompressed-geotiff orthorectified imagery and corresponding metadata for the entire area.

        - Ortho products shall maintain resolution of original image capture. Imagery captured at one resolution shall not be resampled to a higher image resolution. This shall not prohibit imagery collected at a higher resolution from being resampled to a lower resolution. For example, the area of 10cm resolution imagery may be resampled to produce the coverage to be delivered at 30cm resolution

        - Every frame shall be used in the orthorectification process to minimize radial distortion in imagery.

      - Ground Resolution: The spatial resolution shall be10 or 30 centimeter ground sample distance (GSD). Orthoimagery produced under this specification shall not be resampled from the original image, original scan or original capture, with resolution greater or less than the following numbers:

| Ground Sample | Original Image Resolution |  |  |
| --- | --- | --- | --- |
| Distance (GSD) | Maximum | Minimum |  |
| 30 centimeters | 15 centimeters | 32 centimeters |  |

| Ground Sample | Original Image Resolution |  |  |
| --- | --- | --- | --- |
| Distance (GSD) | Maximum | Minimum |  |
| 10 centimeters | 5 centimeters | 11 centimeters |  |

      - Digital Orthorectified Image Horizontal Accuracy: For 10cm Ortho image horizontal positional accuracy shall not exceed 0.507-meters NSSDA 95% confidence (0.293-meters Root Mean Squared (RMSE) Error XY (0.207 meter RMSE X or Y).

      - Digital Orthorectified Image Horizontal Accuracy: For 30cm Ortho image horizontal positional accuracy shall not exceed 1.52-meters NSSDA 95% confidence (0.88-meters Root Mean Squared (RMSE) Error XY (0.62 meter RMSE X or Y).

      - Digital Orthorectified Image Format: Images shall be submitted in uncompressed, untiled, ArcGIS readable, GeoTIFF file format with no internal tiling or overviews.  Data shall not be compressed during ANY PHASE of the production process.  Presence of compression artifacts will be cause for rejection.  GeoTIFF files shall include the following GeoTIFF tags and keys:

        - ModelTiepointTag

        - ModelPixelScaleTag

        - GTModelTypeGeoKey

        - GTRasterTypeGeoKey

        - ProjectedCSTypeGeoKey

        - PCSCitationGeoKey

        - ProjLinearUnitsGeoKey

      - Digital Orthorectified Image Datum - UTM (1foot Tiles ONLY):  Digital Orthorectified images shall be referenced to UTM Zone 11.

        - Digital Orthorectified Image Tile Size: Orthorectified GeoTIFF files shall represent “tiles” 1500 meters X 1500 meters with no tile overedge.

      - File Naming Convention:

        - UTM: The 1500 meter x 1500 meter ortho tile file name shall be derived from the US National Grid Tiling Scheme.

      - Digital Orthorectified Image Datum - STATE PLANE (1 foot and 4in resolution tiles):  Digital Orthorectified images shall be referenced to California State Plane Zone VI, survey feet.

        - Digital Orthorectified Image Tile Size (1 foot): Orthorectified GeoTIFF files shall represent “tiles” 5000 feet X 5000 feet with no tile overedge.

        - Digital Orthorectified Image Tile Size (4 inch): Orthorectified GeoTIFF files shall represent “tiles” 2500 feet X 2500 feet with no tile overedge.

      - File Naming Convention:

        - State Plane: Will be according to Digital Ortho Base Specification v1.0.

      - Horizontal Accuracy: All orthoimagery shall have 95% (NSSDA Confidence Interval) of all well-defined points tested fall within the specified distance listed below of true ground:

        - 10cm GSD shall be 0.51 meters (or feet equivalent)

        - 30cm GSD shall be 1.52 meters

      - Product Accuracy Information Reporting.  Product accuracy information shall be reported according to NSSDA guidelines which are available at: http://www.fgdc.gov/standards/projects/FGDC-standards-projects/accuracy/part3/index.html

      - At a minimum, statements concerning source materials and production processes used must be provided at the project level sufficient to meet the requirement of section III.E of the guidelines.

      - Elevation data: Elevation data created for use in the orthorectification process shall be submitted, with associated documentation, in a common or non-proprietary format agreed to by the government.

      - EXCLUSION AREA: The area defined as Military Exclusion Area, within the San Diego upgrade footprint shall be excluded from public domain delivery of the 4” Upgrade Imagery and Lidar Point Cloud Data. (2.5-foot DEM deliverable is not restricted)

    - FLIGHT DIAGRAM:  The Contractor shall produce, in softcopy format, a Flight Diagram which illustrates the project area outline, flight lines, image identification, and approximate location and Time Stamp of image centers.

    - Project Deliverable Shape File: The Contractor shall produce, in softcopy format, a shape file showing delivered tiles for this project. Attributes of each tile in the shape file shall include, but shall not be limited to:

      - Tile Name

      - Date(s) of Image Acquisition of imagery used in tile

    - METADATA: Project and file metadata describing the orthophoto production process shall be submitted as a deliverable.

      - Federal Geographic Data Committee (FGDC) compliant metadata shall be provided in extensible markup language (.xml) format for each 1500-meter x 1500-meter orthorectified tile (UTM one foot), each 5000-foot x 5,000-foot tile and each 2500-foot x 2500- foot tile.

      - FGDC compliant metadata for orthoimage tiles shall be delivered on portable media, in the file format mutually agreeable to the Government and the Contractor.

      - This site contains the files designed to define and support production of FGDC compliant metadata: Download the following files from ftp://ftpext.usgs.gov/pub/cr/mo/rolla/release/xmlinput/

        - XMLInput1_64.zip: Contains an application (XMLInput) for creating and editing .xml metadata files. It is not mandatory that this software is used; it is merely available to you. When the zip file is unzipped, it also contains a template (133UAtemplate.xml) and a dtd (csdgm2.dtd) to help with FGDC compliance. The XMLInput.jar is the executable.

        - Help.pdf and XMLInput123.doc: User’s guide for XML Input. Use this guide to install and use XMLInput.

        - metadata _overview.doc: Additional information.

        - Metadata Parser (MP): Is a compiler that checks for syntax errors against the FGDC Content Standard for Digital Geospatial Metadata and generates output suitable for viewing in a web browser or text editor.  MP may be downloaded to your computer for an interactive validation of your metadata files, or accessed online for an automated validation.

          - To download mp, go to http://geology.usgs.gov/tools/metadata/

          - To use mp online, go to http://geo-nsdi.er.usgs.gov/validate.php

    - LIDAR QL2 Data Production and Handling: The contractor shall be responsible for post processing of lidar data of sufficient density and quality to meet the requirements specified in the referenced Version 1 specification. All processing should be carried out with the understanding that all point deliverables are required to be in fully compliant LAS format, v1.2 or v1.3. Data producers are encouraged to review the LAS specification in detail. Specifications of the LAS data sets will be verified.

      - ACCURACY REPORTING

        - Data Accuracy: Data collected under this Task Order shall meet the National Standard for Spatial Database Accuracy (NSSDA) accuracy standards.  The NSSDA standards specify that vertical accuracy be reported at the 95 percent confidence level for data tested by an independent source of higher accuracy.  For example the metadata statement shall read, “Tested __ (meters, feet) vertical accuracy at 95 percent confidence level.”

          - Accuracy of the Lidar Point Cloud Data: The Fundamental Vertical Accuracy (FVA) of the Lidar Point Cloud data shall be calculated against TINs derived from the final calibrated and controlled swath data. The required accuracy (ACCZ) is: 18.5 cm at a 95% confidence level, derived according to NSSDA, i.e., based on RMSE of 9.25 cm in the “open terrain” land cover category. This is a required accuracy.

          - Accuracy of the Derived DEM: The accuracy (ACCZ) of the derived DEM shall be calculated and reported in three (3) ways:

- Fundamental Vertical Accuracy (FVA): The required FVA is: 18.5 cm at a 95% confidence level, derived according to NSSDA, i.e., based on RMSE of 9.25 cm in the “open terrain” land cover category. This is a required accuracy.

- Supplemental Vertical Accuracy (SVA): SVAs shall be reported for each of the land cover classes identified in C.1.b.(vii)(b) above. The target SVA is: 26.9 cm at a 95th percentile level, derived according to ASPRS Guidelines, Vertical Accuracy Reporting for LiDAR Data, i.e., based on the 95th percentile error for each required land cover class. These are target accuracies.

- Consolidated Vertical Accuracy (CVA): The required CVA is: 26.9 cm at a 95th percentile level, derived according to ASPRS Guidelines, Vertical Accuracy Reporting for LiDAR Data, i.e., based on the 95th percentile error in all land cover categories combined. This is a required accuracy.

      - Hydro Flattening Requirements:

        - Inland Ponds and Lakes:

          - ~2-acre or greater surface area (~350’ diameter for a round pond)

          - Flat and level water bodies (single elevation for every bank vertex defining a given water body).

          - The entire water surface edge must be at or just below the immediately surrounding terrain.

          - Long impoundments such as reservoirs, inlets, and fjords, whose water surface elevations drop when moving downstream, should be treated as rivers.

        - Inland Streams and Rivers:

          - 100’ nominal width: This should not unnecessarily break a stream or river into multiple segments. At times it may squeeze slightly below 100’ for short segments. Data producers should use their best professional judgment.

          - Flat and level bank-to-bank (perpendicular to the apparent flow centerline); gradient to follow the immediately surrounding terrain.

          - The entire water surface edge must be at or just below the immediately surrounding terrain.

          - Streams should break at road crossings (culvert locations). These road fills should not be removed from DEM. However, streams and rivers should not break at bridges. Bridges should be removed from DEM. When the identification of a feature as a bridge or culvert cannot be made reliably, the feature should be regarded as a culvert.

        - Non-Tidal Boundary Waters:

          - Represented only as an edge or edges within the project area; collection does not include the opposing shore.

          - The entire water surface edge must be at or below the immediately surrounding terrain.

          - The elevation along the edge or edges should behave consistently throughout the project. May be a single elevation (i.e., lake) or gradient (i.e., river), as appropriate.

        - Tidal Waters:

          - Water bodies such as oceans, seas, gulfs, bays, inlets, salt marshes, very large lakes, etc. Includes any significant water body that is affected by tidal variations.

          - Tidal variations over the course of a collection, and between different collections, will result in discontinuities along shorelines. This is considered normal and these “anomalies” should be retained. The final DEM should represent as much ground as the collected data permits.

          - Variations in water surface elevation resulting in tidal variations during a collection should NOT be removed or adjusted, as this requires either the removal of ground points or the introduction of unmeasured ground into the DEM. The USGS NGP priority is on the ground surface, and accepts the unavoidable irregularities in water surface.

          - Scientific research projects in coastal areas often have very specific requirements with regard to how tidal land-water boundaries are to be handled. For such projects, the requirements of the research will take precedence.

    - ORTHO DELIVERABLES SUMMARY:

      - Calibration Reports:  Camera Calibration Report(s) for Aerial Camera(s), or in the case of digital sensors, a current Product Characterization Report of the instrument used shall be included as a deliverable.

      - Camera Station Control:

        - Airborne GPS: Positional data and statistical summary report shall be submitted on portable media, in a non-proprietary format mutually agreeable to the Government and the Contractor. The ABGPS report shall also include x, y, z, omega, phi, kappa, and time stamp for each frame

        - IMU Data:  If IMU exterior orientation data is part of the Contractors Technical Proposal, the Contractor shall submit this sensor orientation data and a statistical summary report on portable media, in a non-proprietary format mutually agreeable to the Government and the Contractor.

      - Digital Stereo Pairs shall be provided for 10cm and 30cm imagery.  Images shall be in GeoTiff format.

      - MrSID Images mosaics shall be generated for the 30cm imagery based on a compression ratio of 15:1 and using MrSID Gen 3 format.  Image mosaics shall NOT exceed 2 GB in size for any single image. MrSID Images are NOT required for 10cm imagery.  Compressed data to be in California State Plane Zone VI, survey feet.

        - Accuracy Specifications:

          - Airborne GPS: The horizontal root-mean-square error (RMSE) of the airborne GPS control data shall not exceed 20cm.  The vertical RMSE of the Airborne GPS control shall not exceed 30cm.

          - IMU: If IMU is included as a component of the camera station control; the contractor shall record the camera attitude at the instant of exposure. The IMU data shall be adjusted and organized as individual data sets grouped by corresponding flight. The RMSE of the adjusted IMU data shall not exceed 30cm.

        - Supplemental Ground Control: Differentially corrected GPS ground control, or conventionally surveyed first-order ground control, used to supplement the Airborne GPS positional adjustment shall be stored on portable media, in a non-proprietary format mutually agreeable to the USGS and the cooperator.  The data provider shall publish and submit a Supplemental Ground Control report that contains narrative, computations and field notes/photos for all points used in the supplemental ground control solution. 20 to 30 points are expected to be collected for 30cm resolution imagery. An additional 20 points shall be collected for the 10cm imagery.

      - Aerotriangulation Report: Aerotriangulation (AT) data, if used in the orthorectification process, shall consist of a minimum of refined image coordinates and adjusted ground coordinates.  If Aerotriangulation is performed, the data provider shall provide a comprehensive AT report.

      - Flight Diagram: A Flight Diagram that illustrates the project area outline, the location of the flight lines and the approximate location of image centers, if relevant, shall be included as a deliverable.  This diagram shall be provided in softcopy.

      - Digital Orthorectified Image Format: Images shall be submitted in uncompressed, untiled, ArcGIS readable, GeoTIFF file format, Version 1.8.2, (http://www.remotesensing.org/geotiff/spec/geotiffhome.html) with no internal tiling or overviews. Data shall not be compressed during ANY PHASE of the production process. Presence of compression artifacts will be cause for rejection. GeoTIFF files shall include (as a minimum) the following GeoTIFF tags and keys:

        - ModelTiepointTag

        - ModelPixelScaleTag

or

        - ModelTransformation Tag

and

        - GTModelTypeGeoKey

        - GTRasterTypeGeoKey

        - ProjectedCSTypeGeoKey

      - Metadata shall be FGDC compliant and shall be project and tile (file) level. Tile level metadata shall have one corresponding metadata file for each imagery.

      - Production, Project, and Supplemental Reports as specified.

    - LIDAR DELIVERABLES SUMMARY: The following deliverable products shall be produced from the lidar produced above.

      - Raw Point Cloud Data:

        - Fully compliant LAS v1.2 or v1.3, Point Record Format 1, 3, 4, or 5

        - LAS v1.3 deliverables with waveform data are to use external “auxiliary” files with the extension “.wdp” for the storage of waveform packet data. See the LAS v1.3 Specification for additional information.

        - Georeference information included in all LAS file headers

        - GPS times are to be recorded as Adjusted GPS Time, at a precision sufficient to allow unique timestamps for each return.

        - Intensity values, 16-bit, Linear Rescaling

        - Full swaths, all collected points to be delivered.

        - 1 file per swath, 1 swath per file, file size not to exceed 2GB, as described in Section II, Paragraph 5.

      - Classified Point Cloud:

        - Fully compliant LAS v1.2 or v1.3, Point Record Format 1, 3, 4, or 5, including “File Source ID.”

        - LAS v1.3 deliverables with waveform data are to use external “auxiliary” files with the extension “.wdp” for the storage of waveform packet data. See the LAS v1.3 Specification for additional information.

        - Georeference information included in LAS header

        - GPS times are to be recorded as Adjusted GPS Time, at a precision sufficient to allow unique timestamps for each return.

        - Intensity values, 16-bit, Linear Rescaling

        - Tiled delivery, without overlap

        - Classification Scheme (minimum):

          - Code 1 – Processed, but unclassified

          - Code 2 – Bare-earth ground

          - Code 7 – Noise (low or high, manually identified, if needed

          - Code 9 – Water

          - Code 10 – Ignored Ground (Breakline Proximity)

          - Code 17 – Bridge Decks

        - Note: Class 7, Noise, is included as a convenience for the data producer. It is not required that all “noise” be assigned to Class 7.

        - Note: Class 10, Ignored Ground, is for points previously classified as bare-earth but whose proximity to a subsequently added breakline requires that it be excluded during Digital Elevation Model (DEM) generation.

      - Bare Earth Surface (Raster DEM):

        - Cell Size shall be 2.5 feet (to agree with 5000 foot tile size), and no less than the design Nominal Pulse Spacing (NPS).  Elevations shall be FEET.

        - Delivery in an industry-standard, GIS-compatible, 32-bit floating point raster format (ERDAS .IMG preferred)

        - Georeference information shall be included in raster file

        - Tiled delivery, without overlap

        - DEM tiles will show no edge artifacts or mismatch

        - Void areas (i.e., areas outside the project boundary but within the tiling scheme) shall be coded using a unique “NODATA” value. This value shall be identified in the appropriate location within the file header.

        - Vertical Accuracy (RMSEZ) of the bare earth surface is to be assessed using the methods described in the FEMA “Guidelines and Specifications for Flood Hazard Mapping Partners, Appendix A”, Section A.8.5 paragraph 1, Section A.8.6.1, and Section A.8.6.2 (substituting the contracted vertical accuracy requirements (RMSEZ) for those listed in the FEMA document). All QA/QC analysis materials and results are to be delivered to the USGS.

        - Depressions (sinks), natural or man-made, are not to be filled (as in hydro-conditioning and hydro-enforcement).

        - Water Bodies (ponds and lakes), wide streams and rivers (“double-line”), and other non-tidal water bodies as defined in Section III are to be hydro-flattened within the DEM. Hydro-flattening shall be applied to all water impoundments, natural or man-made, that are larger than ~2 acre in area (equivalent to a round pond ~350’ in diameter), to all streams that are nominally wider than 100’, and to all non-tidal boundary waters bordering the project area regardless of size. The methodology used for hydro-flattening is at the discretion of the data producer.

        - Contours: Contours at an interval of Two (2) feet shall be delivered. These contours shall be computer generated with no cartographic enhancement, but shall be as generated. Contours shall be delivered in ArcGeodatabase format and shall be tiles to match the DEM deliverable.

      - Control: Control, as defined above shall be delivered to the Government as specified in Digital Deliverables.

      - Metadata: The following requirements for Metadata shall be met:

        - Collection Report detailing mission planning and flight logs.

        - Survey Report detailing the collection of control and reference points used for calibration and QA/QC.

        - Processing Report detailing calibration, classification, and product generation procedures including methodology used for breakline collection and hydro-flattening.

        - QA/QC Reports (detailing the analysis, accuracy assessment and validation of:

          - The point data (absolute, within swath, and between swath)

          - The bare-earth surface (absolute)

          - Other optional deliverables as appropriate

        - Control and Calibration points: All control and reference points used to calibrate, control, process, and validate the lidar point data or any derivative products are to be delivered.

        - Geo-referenced, digital spatial representation of the precise extents of each delivered dataset. This should reflect the extents of the actual lidar source or derived product data, exclusive of Triangular Irregular Network (TIN) artifacts or raster NODATA areas. A union of tile boundaries or minimum bounding rectangle is not acceptable. ESRI Polygon shapefile is preferred.

        - Product metadata (FGDC compliant, XML format metadata). One file for each:

          - Project

          - Lift (note: this one per lift – not one per project)

          - Tiled deliverable product group (classified point data, bare-earth DEMs, breaklines, etc.). Metadata files for individual tiles are not required.

      - Project Report: The contractor shall deliver a production report which details:

        - A record of field work procedures.

        - Data derivation and adjustments.

        - Quality control procedures and results.

        - Any problems encountered and solutions used in resolving such problems.

        - Statistical report summarizing the results of the airborne GPS adjustment and the overall accuracy of the adjusted IMU data.

        - Production report shall be Microsoft Word, Adobe PDF format or other compatible digital format.

      - Acquisition Reports: Contractor shall provide regular progress updates to the technical point of contact throughout the data acquisition process.

        - Update frequency shall be based upon the collection period, but no less that once a week.

        - Reports shall be delivered as shapefiles which represent the geographic extent of the acquired data.

      - Updates shall commence at acquisition onset and shall continue until acquisition is complete\

      - LIDAR TILING SCHEME AND DATA FORMAT

      - Tile Coverage: Tiles which lie completely within the project area shall be complete to the tile edges. Tiles which lie partially outside the project boundary shall be complete to the project boundary with enough overlap beyond the project boundary to ensure that no parts of the project are omitted.

        - Tile Size:

          - Tiles shall be 5000 feet x 5000 feet

          - Tiled deliverables shall conform to the tiling scheme, without added overlap.

          - Tiling scheme will be used for all tiled deliverables.

          - Tiled deliverables shall edge-match seamlessly in both the horizontal and vertical.

        - Tile Naming:

          - Tiles shall be named according to the US National Grid conventions.

        - Spatial Reference System: The Spatial Reference System shall be: State Plane CA Zone VI, survey feet (NAVD88 using the latest geoid model available from the National Geodetic Survey)

    - NOTIFICATION: The Government POC named below shall be notified within 24 hours of the start of acquisition of data. Notification can be made by e-mail and is for information purposes only, not permission to proceed.

    - PERMITS: The contractor shall be responsible for obtaining all permits which may be required in the performance of this task order, which shall include, but not be limited to any permits for acquisition of data in controlled or restricted airspace, and access to control points on the ground.

    - USE AND DISTRIBUTION RIGHTS: All imagery and data produced under this agreement shall become the property of the United States Government. All data and documentation shall be free from restrictions regarding use and distribution. Data and documentation shall be freely distributable by government agencies.

    - CERTIFICATIONS: The contractor shall certify as part of its proposal that the work performed on this task order complies with Section 52.225-05 of the Federal Acquisition Regulations relating to Trade Agreements.

    - THE GOVERNMENT POINT-OF-CONTACT (POC) for this task order is:

- Address: USGS/NGTOC

- ATTN:                                                                  Telephone:(573) 308-3689

- Dan Vincent	FAX:(573)-308-3810
             Rolla Federal Center	e-mail: dvinc@usgs.gov
             MS 664

- 1400 Independence Road
             Rolla, MO 6540

  - Digital Deliverables: Reference 3.11 of the Contract.

    - The Contractor shall deliver the following copies of the Ortho Image data products and documentation as specified in Section C.1 of this Task Order.

      - 10cm imagery: Two (2) complete set of imagery on external hard drives.

      - 30cm imagery: Two (2) completed sets of imagery on external hard drives. This shall include the Mr.Sid products as well.

      - QL2 Lidar: Two (2) complete sets and all associated datasets on external hard drives.

    - Format: Data shall be delivered in the formats specified in C.1 above.

    - Delivery Medium: The digital data shall be delivered on external hard drive, i.e. (firewire, or USB2 – Less than USB2 is not acceptable).

    - Deliverable Validation: Reference 3.12 of the Contract. The Government may choose to contract with a separate contractor for validation on all submitted deliverables.

- - PACKAGING AND MARKING

  - No additional Section D requirements are applicable to this Task Order.

- - INSPECTION AND ACCEPTANCE - The following Section E additional requirements are applicable to this Task Order:

  - Inspection Period: Reference GS0720 of the Contract. The inspection period begins the day after the data has been delivered. All deliverables will be validated within a sixty (60) calendar-day inspection period

  - Inspection and Acceptance Procedures: Reference E780 of the Contract. The Government will perform a full inspection of all deliverables in accordance with E780 (b) of the Contract.

  - Nonconforming deliverables: Nonconforming deliverables returned to contractor for rework shall be delivered in accordance with Contract clause E784.

- - DELIVERIES OR PERFORMANCE - The following Section F additional requirements are applicable to this Task Order:

  - Place of Delivery: Reference GS0904 of the Contract. Contractor shall submit all requested deliverables to the address of the POC, as shown in Section C of this Task Order.

  - Proposed Delivery Schedule: Reference F981 of the Contract. The Government requires the following delivery schedule:

    - Delivery Lot One (1) Kick-off Meeting: A kick-off meeting shall be held to outline communication procedures that will be followed for data acquisition with respect to verification of local ground conditions and vegetation requirements. The kick-off meeting will be held no later than two (2) weeks after contract award and prior to data acquisition, but no later than October 10, 2014.

    - Delivery Lot Two (2): Imagery Acquisition: Imagery Acquisition will be completed by December 31, 2014, airspace access, weather and ground conditions permitting.

    - Delivery Lot Three (3): LiDAR Acquisition: LiDAR Acquisition will be completed by December 31, 2014, airspace access, weather and ground conditions permitting.

    - Delivery Lot Four (4): Ground Control: Control surveys to support both ortho imagery and LiDAR processing will be completed by December 1, 2014.

    - Delivery Lot Five (5): Digital Stereo Pairs: 10cm and 30cm Digital Stereo Image Pairs will be delivered 30 calendar days following completion of Image Acquisition, but not later than January 30, 2015.

    - Delivery Lot Six (6): Pilot Lidar Data - Consisting the FVA reporting of the UNCLASSIFIED point cloud data, and a minimum of five (5) square miles of classified LAS data in each of the primary land cover categories (SVA categories). This pilot data will be delivered no later than February 15, 2015.

    - Delivery Lot Seven (7): 10cm Digital Orthoimagery: Consisting of 4 band (R,G,B,NIR), 8 bit orthorectified, tiled, GeoTiff imagery at 10cm (4”) GSD resolution, tiled imagery, metadata, reports, and other associated deliverables will be delivered 90 calendar days following completion of imagery acquisition, but no later than April 15, 2015.

    - Delivery Lot Eight (8): 30cm Digital Orthoimagery: Consisting of 4 band (R,G,B,NIR), 8 bit orthorectified, tiled, GeoTiff imagery at 30cm (1’) GSD resolution, tiled imagery, metadata, reports, and other associated deliverables will be delivered 30 calendar days after USGS acceptance of  Lot 7 but no later than May 31, 2015.

    - Delivery Lot Nine (9): LiDAR Data: Consisting of all required deliverables (including metadata) of the LiDAR data and its derived products as specified in the task order, will be delivered no later than 60 calendar days following Government response to Pilot Data, but in no case later than May 31, 2015.

    - Delivery Lot Ten (10): Final Data Set Copies: Consisting of required copies of all final and fully USGS accepted ortho and LiDAR datasets and associated documentation, reports, etc. per Task Order requirements will be delivered within 14 days after final USGS acceptance of each ortho imagery data set (Lots 7 and 8) and LiDAR (Lot 9) data identified above but no later than August 1, 2015.

  - Progress Reports: Contractor shall submit a monthly progress report for this task order in accordance with Contract clause GS0921 and GS0931.

- - CONTRACT ADMINISTRATION DATA

  - No additional Section G requirements are applicable to this Task Order

- - SPECIAL CONTRACT REQUIREMENTS -The following Section H additional requirements are applicable to this Task Order:

  - Applicable Regulations And Permits -- Aircraft Operations:  Reference H1344 of the contract.  The contractor shall be responsible for applying for and obtaining any required permits for access, over-flight, or intrusion into restricted or otherwise limited ground access and/or airspace, which may be included within the requirement of this task order.

  - Government Furnished Property: Reference GS1434 1 (c) of the contract. No Government furnished property is being supplied with this Task Order.

- - CONTRACT CLAUSES

  - No additional detail is required for this Task Order.

- - LIST OF ATTACHMENTS TO THIS TASK ORDER

  - Attachment A - 	Project Descriptions/Diagrams  	                     3 Pages

  - Attachment B -                 Project Deliverables Matrix/Diagrams                    5 Pages

  - Attachment C - 	Project Datasets	                     1 Page

  - Attachment D -	Military Exclusion Areas	                     1 Page

TASK ORDER Attachment A -

San Diego CA Ortho Image Task Order – Project Description and Diagram

Portion of San Diego County 30cm (nominally 1-foot) GSD Collection Area.

TASK ORDER Attachment A -

San Diego CA Ortho Image Task Order – Project Description and Diagram

San Diego County 10cm (nominally 4 inch) GSD Collection Area and QL2 Lidar.

TASK ORDER Attachment A -

San Diego CA Ortho Image Task Order – Project Description and Diagram

Camp Pendleton 10cm (nominally 4 inch) GSD Collection Area and QL2 Lidar.

END “ATTACHMENT A”

TASK ORDER Attachment B -

San Diego CA Ortho Image Task Order – Project Deliverables Matrix and Diagram

Portion of San Diego County 30cm (nominally 1-foot) GSD Collection Area.

TASK ORDER Attachment B -

San Diego CA Ortho Image Task Order – Project Deliverables Matrix and Diagram

Portion of San Diego County 30cm (nominally 1-foot) GSD Collection Area.

TASK ORDER Attachment B -

San Diego CA Ortho Image Task Order – Project Deliverables Matrix and Diagram

Portion of San Diego County 30cm (nominally 1-foot) GSD Collection Area.

TASK ORDER Attachment B -

San Diego CA Ortho Image Task Order – Project Deliverables Matrix and Diagram

San Diego County 10cm (nominally 4 inch) GSD Collection Area and QL2 Lidar.

TASK ORDER Attachment B -

San Diego CA Ortho Image Task Order – Project Deliverables Matrix and Diagram

Camp Pendleton 10cm (nominally 4 inch) GSD Collection Area and QL2 Lidar.

END “ATTACHMENT B”

TASK ORDER Attachment C -

San Diego CA Ortho Image Task Order – Additional Attached Files

THIS SECTION CONSISTS OF THE FOLLOWING DATA SET(S)

Project Shape Files:

Portion_of_San_Diego_one_foot.zip

San_Diego_4inch_and_QL2_LiDAR.zip

Camp_Pendelton_4inch_and_QL2_LiDAR.zip

Military_Exclusion_Areas.zip

one_foot_compressed_mosaic_outlines.zip

END “ATTACHMENT C”

TASK ORDER Attachment D-

San Diego CA Ortho Image Task Order – Military Exclusion Areas

END “ATTACHMENT D”